Lab System Procedure (LSP)

Ensuring Validity of Test Results (QA)

#### ISO/IEC 17025:2017 – Clause 7.7

#### Copyright

The copyright of this procedure, together with all confidential information contained herein is the sole property of QC LAB.

It may be copied in full or in parts only by the Management / personnel and only for the purpose of Company related activities. Disclosure and copying of any information contained within this procedure to any person (s) outside the employee of company without written permission/authorization of the Plant Manager is strictly prohibited.

| Prepared By: AQCM | Reviewed By: QCM | Approved By: PLANT MANAGER |
| --- | --- | --- |

| Amendment Sheet |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | Rev # | DCR # | Section | Amendment Detail | Amendment Approved by |

1. Purpose

The purpose of this document is to ensure that all testing activities carried out at QC lab of VAN Industries (HI) meets the requirement to assure the quality of the test result & verify That those results are accurate & precise.

2. Scope

This procedure is applicable to Quality Control laboratory where testing is being conducted

3. Responsibility

Quality Control Manager is responsible for

- For execution of this procedure

- Ensure the coordination for proficiency testing (PT)

- Approve Quality Assurance Program for lab

- Ensure that developed program is followed

- Approve list of selected PT providers

- Lab Registration with finalized PT providers

- Develop quality assurance plan for the lab

- Develop list of selected PT providers

- Ensure execution of the plan & maintain documentary evidence of its implementation.

- Compile & store all the results & records that are generated from quality assurance activities

4. Procedure

In order to assure the quality of the test being performed, lab management is to make concerted efforts to ensure strict compliance through Quality Assurance Program. Lab Quality Control is essential aspect of ensuring that data released is fit for the purpose determined by the quality control objectives (ie. accuracy & precision). Lab Quality Assurance program includes:

- Participation in Proficiency Testing (PT)

- retesting or recalibration of retained items

- replicate tests or calibrations using the same or different methods

- Estimation of uncertainty

- Use of Certified Reference Material

- use of alternative Instrumentation that has been calibrated to provide traceable results;

- correlation of results for different characteristics of an item

- functional check(s) of measuring and testing equipment;

- use of check or working standards with control charts, where applicable;

- intermediate checks on measuring equipment:

- testing of blind sample(s).

- Participation in Inter-lab Comparison (ILC)

- Review of reported results

- Intralaboratory comparison

4.1. Proficiency Testing (PT)

4.1.1 Selection process for PT providers

Quality Control Manager is primarily accountable for searching PT providers especially the lab tests in the accreditation scope. Quality Control Manager shortlists the PT providers & inform initial contact via email, fax or telephone. In case of more than one PT provider then Quality Control Manager in consultation with PM selects the PT, provider considering the following:

- Recognition of the PT providers by national & international bodies Le. APLAC ILAC, EA, member of ILAC & APLAC

- Financial considerations

- Appropriateness of the PT scheme to parameters in the scope of accreditation

- Suitability of program (Time, parameter)

Note: Quality Assurance program will be developed in accordance with schedule provided by the PT provider.

4.1.2 Registering in the PT program

Manager QC is responsible to identify PT providers for tests in the scope of the accreditation. Upon finalization of PT provider, Quality Control Manager enters its details in the list of proficiency test providers & make preparations as per the instructions provided by the PT provider

4.1.3 Managing PT process for participation & test results submission

- Upon receiving blind samples, Quality Control Manager inspect the following before accepting it:

- Packaging of the sample for any physical damage & tempering

- Sample container, & sample itself for any physical/visible damage, leak tampering etc.

- In case of any non-conformity, PT provider is Immediately notified & test on the sample is not performed until &unless PT provider gives a go-ahead or re-issue a new sample (this step is also performed in case sample is damaged by the lab itself).

- If the sample is found to be in acceptable state, its acceptance is reported to PT provider by Quality Control Manager, while ensuring that date is noted to estimate the total time available for performance of test & delivery of test results (Time line is provided by PT service provider)

- Quality Control Manager ensures the following before performance of the test on blind samples is initiated:

- Analyst to perform the test is identified on the basis of his/her authorization & expertise (supervisor level recommended)

- Test method available

- Equipment is ready

- Assistant Chemist ensures the following before starting the test performance

- Availability of lab note book for documentation of activity

- Availability of appropriate test method

- Equipment & required additional apparatus is ready for use, & work instruction available

- Environmental conditions (if any) are in compliance with test requirements & not exceeding the defined limits.

- Any additional documents or forms required for the test are available (based on test requirements by PT service provider)

- When everything mentioned above is confirmed & ready. Chemist starts the performance of the test

- Quality Control Manager ensure monitoring while the test is being performed & ensure fulfillment of required documentation for the submission of the test to PT provider

- Will ensure that the results are documented as per PT provider instructions

- The results are communicated to PT provider utilizing the medium of service provider's preference

- Remaining sample is stored in a controlled environment for future references & activities (when possible)

4.1.4 PT Result Analysis

- After receiving the results from PT service provider, it is determined whether QA/QC results have been considered "satisfactory" or "Non-satisfactory (Questionable)"

- No further actions required in case of satisfactory results, Quality Control Manager can now initiate new programs for PT, or start communication for next due round

- In case of unsatisfactory results, lab will follow these steps:

- Quality Control Manager Can Initiate CPA form & investigate for Identification of root cause behind the issue following "procedure for Corrective & Preventive Action".

- Quality Control Manager also involves IT providers to collect their expert opinion

- Based on the result of the investigation, Manager QC decides after taking opinion from Managing Director to rectify the problem as well as re-initiates PT process

- In case of unsatisfactory results of investigation (unable to discover the actual cause of unsatisfactory test results), Quality Control Manager will register PT program with a new service provider

4.2. Retesting & Replicate Testing of samples

4.2.1 Retesting

Retesting is a planned activity & performed at periodic intervals to verify previously performed tests. It must be understood here that "retesting a sample because of non- conformity or because of an ongoing investigation or compliant" is not the same. ISO 17025 requires for retesting of samples. All such samples are kept in storage under acceptable conditions. The intention is to get the same result as of the first performance. It can also be adopted in cases; an analysis is not giving the required result, In Short:

"Retesting means same person performing the same test repeatedly under same conditions"

4.2.2 Replicate Testing

Replicate testing is also a planned activity at periodic intervals to verify previously performed tests. It is performing tests on a volume of sample after replicating it & distributing it to different staff members. The intention is to get the same results out of each of them, but lab supervisors must ensure that all samples replicates carry the same characteristics/In short:

"Replicate testing means different persons performing same tests under same conditions"

Replicate / Retesting will be performed as per formulated Quality Assurance Program/Plan. Retesting is performed by rechecking results by the same method &chemist whereas Replicate Testing is performed by rechecking results by the same method but by a second chemist.

a. Selection of personal

Before finalizing lab staff members to contribute in this activity Quality Control Manager should ensure the following:

- Performer is fully trained in the selected testing activity & has training records available to justify this statement

- Performer is available on the required date/time for performance

- He has good practice on the equipment/apparatus being used

- He understands the requirements of replicate/ retesting activities & its importance in lab management system

- He is capable of providing data that is relevant to testing report

- He is capable of highlighting weak areas & also can address corrective actions

b. Sample preparation & distribution control

Quality Control Manager is responsible for finalizing sample preparation procedure & shall appoint a person who perform the activities to mark each sample a unique ID, eg. REP-TEST-01/RETEST-01. REP-TEST-02/ RETEST-02, etc. This data shall be recorded in Replicate Testing/Retesting Report, so that related data is available at one place

c. Result compilation & analysis

After collection of results from staff member involved, Quality Control Manager should develop Replicate/ Retesting report which should include the following:

- Dates/Time

- Name of the personal Involved

- Sample results with their unique ID's

- Calculation of Average (based on Results)

- Standard Deviation (based on results)

d. Corrective action & follow up

In case results or any condition varies & difference at some level is very high, activity should be rescheduled for the same type of sample. If the difference persists then corrective action should be initiated to address the problem.

4.2.3 Estimation of Uncertainty It is the uncertainty of the personnel performing the test, as personnel's error it cannot be calculated in the form of numerical values but estimated by determining the combined estimation of Repeatability & Reproducibility through the test results

5. Related Documents

- Procedure for Estimation of uncertainty (QCL-LSP-13)

- PT Correspondence & Communication File

- Quality Assurance (QA) Program (QCL-FRM-16.01)

- Repeatability & Reproducibility Report (QCL-FRM-16.02)

- List of proficiency Testing providers (QCL-FRM-16.03)

- PT Performance Register (QCL-REG-16.04)