Procedure for Handling of Test and Calibration Items

ISO/IEC 17025:2017 Clause 7.4

| Prepared By: Masab Rasheed | Reviewed By: Himayat Hussain | Approved By: Ahmed Umair |
| --- | --- | --- |
| AQCM | QCM | CEO |
| Date: | Date: | Date: |

#### Copyright

#### The copyright of this procedure, together with all confidential information contained herein is the sole property of QC LAB.

It may be copied in full or in parts only by the Management / personnel and only for the purpose of Company related activities. Disclosure and copying of any information contained within this procedure to any person (s) outside the employee of company without written permission/authorization of the Chief Executive Officer is strictly prohibited.

Amendment Sheet

| Date | Revision # | Amendment Detail | Section | DCR | Amendment Approved by |
| --- | --- | --- | --- | --- | --- |
| 05-02-25 | 01 | Addition of Acceptance criteria and appropriate information in the procedure | Document | 06-25 | Chief Executive Officer |

PURPOSE:

The purpose of this document is to define the process for handling of test and calibration items and conclusions as prescribed by the organization.

SCOPE:

This procedure is applicable for VAN QC Lab’s transport, receipt, handling, protection, storage, retention, and disposal or return of test items.

PROCEDURE:

### Acceptance Criteria for Sample Receiving:

The laboratory will only accept samples that meet the following requirements:

- Proper Condition

  - Samples must be well-packaged, undamaged, and uncontaminated.

- Complete Information

  - The submission form must include all necessary details, such as:

    - Customer name and contact information.

    - Sample identification.

    - Collection date and requested tests.

  - Forms with missing or incorrect details will not be accepted.

- Lab Capability

  - The lab must have the equipment, expertise, and resources to handle the requested tests.

  - If the lab cannot perform the test, the sample will not be accepted.

- Equipment Detection Limits

  - Tests must fall within the detection range of the lab’s equipment.

  - If not, the sample will be rejected.

- Compliance with Rules

  - Samples must follow lab safety and regulatory standards.

  - Non-compliant samples will not be accepted.

- Timely Submission

  - Samples must arrive within the required timeframe to ensure accurate testing.

  - Late or expired samples will be rejected.

- Special Requirements

  - Samples needing special handling (e.g., refrigeration or hazardous materials) must meet the required conditions.

  - If not, they will be rejected.

- Receipt

- QC Lab receives the sample from Customer and signs on the covering document (Customer agreement and analysis request) available with the Lab.

- Before receiving, QC Lab must inspect the samples packaging and physical state.

- Receiving is recorded in the sample receipt register available with QC Lab.

- Transport

  - Lab Representative/Incharge sends the sample to the respective analyst immediately after receiving.

- Handling

- Lab staff receives the sample and records it in the sample log register.

- The samples are placed at their appropriate specified places until initiation of analytical process.

- Perishable samples are placed in the refrigerator while non-perishable samples are placed in dry, isolated storage.

- The samples are homogenized before analysis.

- One part of the homogenized sample is used for analysis.

- Protection

  - The samples have remained within the lab premises throughout its testing/storage cycle.

  - The lab access is limited and specified while the lab remains locked during off hours.

- Retained

- The samples are retained after analysis at specified, isolated and dry place while perishable samples are stored in refrigerator.

- Retention

  - Sample retention period is 15 days after issuance of report.

- Disposal/Discard or return

  - After specified retention period, the samples are disposed of appropriately by lab Attendant and disposal record is maintained. Returning of sample is generally not allowed; however, high value samples could be returned based on prior agreement and receiving must be signed by the client on sample return form (QCL-FRM-22.04)

- Precautions:

The following precautions must be taken to avoid any damage to the sample: -

- The collected sample must be representative of the whole. When the sample is received, the customer should ensure that the sample is homogenized and the aliquot that is going to the lab must be a representative of that whole sample.

- Samples must be stored in a sealed container to avoid any cross-contamination.

- Proper sample records of the test samples and those that need to be returned must be maintained to avoid any loss.

- Storage conditions of the samples must be maintained to avoid any deterioration.

- Related documents

- Customer agreement and Analysis Request 	(QCL-FRM-12.01)

- Assignment, Summary and Review Form 	(QCL-FRM-19.01)

- Sample Return Form 				(QCL-FRM-22.01)

- Sample Discard Register 				(QCL-REG-08)

- Retaining Sample Register 			(QCL-REG-09)