# Standard Operating Procedure for

Sample Transport, Recipient, Handling, Retention and Disposal

#### Copyright

The copyright of this procedure, together with all confidential information contained herein is the sole property of VAN QC LAB.

| Prepared by: AQCM | Reviewed by: QCM | Approved by: PLANT MANAGER |
| --- | --- | --- |

It may be copied in full or in parts only by the Management / personnel and only for the purpose of Company related activities. Disclosure and copying of any information contained within this procedure to any person (s) outside the employee of company without written permission/authorization of the Quality Control Manager or Plant Manager is strictly prohibited.

Amended Sheet

| Date | Revision # | Amendment Detail | Section | DCR | Amendment Approved by |
| --- | --- | --- | --- | --- | --- |

- Purpose:

The purpose of this procedure is to describe the process for transportation, receipt, handling, protection, storage, retention, disposal and return of test and intermediate checks items.

- Scope:

This procedure is applicable to VAN QC LAB.

- Definition, Abbreviation and Concepts:

- VAN – Vital Agri Nutrients

- AQCM- Assistant Quality Control Manager

- VAN QC LAB- Vital Agri Nutrients Quality Control Lab

3.1. Definition:

- Responsibilities and Authorities

- Assistant Quality Control Manager

- Analyst

- Assist. Quality Control Officer

- Procedure:

5.1. Sample Transport for testing (Out Source)

  - 1. AQCM is responsible to ensure consistency and efficiency, sample storage, and transportation procedures must be properly and accurately documented and adhered to by field personnel.*

    - Samples for testing are transported to outsource Laboratory, the analyst make ensure appropriate conditions for the test samples are as follows;

| Material | Preservation | Transportation Time (Max.) | Reference |
| --- | --- | --- | --- |
| Fertilizer Samples | Room Temperature | 24 hours | AOAC |
| Sample Quantity | As per Laboratory Demand |  |  |

5.1.3. Sample dispatched conditions are note in the quality form (VAN-F-39) In case of temperature excursion from the define limit or any damage, the personnel informs department in charge about the deviation in the conditions.

5.1.4. Packing materials should be used and properly chilled during shipment between <8°C by placing enough ice into the cooler along with the samples. Samples and ice should be packed inside the sealed plastic bag or liner to avoid spillage of melted ice while in transit. *

5.1.5. Samples are tightly closed and packed to prevent spillage or breakage. Labels bearing the sample identification, destination must be attached to each container and duplicate sample is store in laboratory as reference sample.

  - Sample Receive  for Testing (In-house  Source)

    - All temperature samples are reached to the laboratory without any delay so that testing is started on time. Receiving temperature of sensitive samples is checked and recorded before starting testing of the samples and noted in VANPFTL-F-36)

    - Analyst of VANPFTL received the samples, inspect the condition of sample & giving unique identification number to each sample and paste sticker and record is maintained on(F-.82)

    - In case of any damage of sample, Analyst is responsible to immediately inform to QC Manager. QC Manager contact with the customer and provide damage report to customer.

  - Handling and Retention Test and Calibration items

    - When the testing sample received in VANPFTLTL, make sure samples don’t physically mix with each other’s, the analyst responsible to give description on each sample:

- Tested Sample

- Quarantine Sample

- Testing Sample

    - Sample storage units are located in areas of the laboratory that are routinely occupied by staff.  Analyst is responsible to maintain temperature of laboratory Sample and noted in QF. (MA-LAB-F- 17).*

    - Tests samples are stored as per recommended conditions, away from direct exposure to sunlight to retain the integrity of the sample and intermediate checks items.

    - The recommended storage conditions and retention period for test sample is as follows

| Sr. # | Sample Type | Storage Conditions | Retention Period |
| --- | --- | --- | --- |
| 1 | Fertilizer | Room Temperature | One Week |
| 2 | Buffers | As per COA | Till Expiry |

    - Above-mentioned conditions are maintained

    - Upon successful completion of this system of checks, Analyst is responsible to note sample in receive log, give unique number, handled and protected to avoid contamination, and stored as they are received. VAN-LAB-F-37.

Analyst is responsible to paste the description of sample

- Date:

- Sample Carrier:

- Sample Receiver:

- Quantity:

- Packing Net Content

- Receive by:

*

5.3.6. The sample requisition form is to be filled (VAN-LAB-F-36), when the sample received from   inter division.

    - Analyst/QCO is responsible to conduct the required test and communicate results after verification and approval of test reports. (VAN-LAB-TR-01)

    - All the authorized personal makes possible to take all precautions to avoid deterioration, contamination, loss or damaged the items during handling, transportation, storing/ waiting and preparation for testing.

    - The Analyst/ QCO should be make possible to retain and store tested sample for two weeks  at room temperature after analysis, for cross check of analytical results and in cause of any customer complaint.

Production line

Laboratory

6.0. Sample Receive Process

Temperature maintain

Sample Requisition

F-17

Report generate

Conduct Analysis

Sample Retention

Discard

7.0. Sample discard

7.1. The VAN QC LAB. is Fertilizer testing laboratory. Sample drain of in environment and note in QF (F-38).

7.2. The laboratory chemicals, regents and testing wastes are collected in separate container and transfer to AT Waste for disposal. All Laboratory waste / CRM/ chemicals are disposing as per 7.2 and maintain record in QF. (F-85)

8.0. Sample Storage.

8.1. Environments and conditions don’t impact so we store sample at room temperature. (F-40).

8.2. The reference samples dispose of as per plan and Analyst is responsible to maintain record F-38).

| Sample Type | Retention Period | Storage condition | Quantity |
| --- | --- | --- | --- |
| Daily Reference Sample | One Year | Room Temperature | 02 |

8.1. VANPFTL committed to maintain, store, monitor and record the specific requirement need for tests, calibration, chemicals and CRM.

9.0. Related Documents

| Reference documents | Document # |
| --- | --- |
| Facility Environment Monitoring | VAN-LAB-F-17 |
| Customer Agreement &Requisition Form | VAN-LAB-F-36 |
| Handling Of Reference And Work Standard | VAN-LAB-F-19 |
| Sample Discard Record | VAN-LAB-F-38 |
| COC | VAN-LAB-F-39 |
| Chemical dispose record | VAN-LAB-F-85 |
| Sample recipient log | VAN-LAB-F-37 |
| Inward register | VAN-LAB-F-84 |
| MSDS | VAN-LAB-MSDS-01 |
| Work Instruction | VAN-WI-02 |
| Sample retention log | VAN-LAB-F-40 |
| Sample Label | VAN-LAB-F-41 |